SPECIALTY — AIRPORT PAVEMENT MARKING SYSTEM

# AirMark

Manufacturer: Ennis-Flint® by PPG (PPG Traffic Solutions, Greensboro NC). HUBSS is the Canadian distributor. IMPORTANT: the AirMark folder bundles three RELATED but DISTINCT products — see Document Library below.

PURPOSE OF THIS DOCUMENT

Source-of-truth reference for HUBSS website product copy. Replaces existing website descriptions that have been flagged as inaccurate or embellished. Doug Bain to review, correct, and sign off — every claim on the website must be supported by a fact listed in this document.

## What it is

AirMark is the preformed thermoplastic pavement marking system specifically engineered for airfield use — taxiways, ramps, aprons, gates, and vehicular airside roads. The 'airside' descriptor is what distinguishes AirMark from airport landside markings (driveways, parking lots, terminal access roads), which use PreMark for Airports.

AirMark is NON-RUNWAY. Runway and primary taxiway markings use a separate liquid waterborne acrylic — WB Airfield / Traffic Paint for Airfields — applied via airless spray. Three distinct products live in the AirMark folder; each has its own page on this section of the document.

## Key features

- Preformed thermoplastic with retroreflective glass beads embedded throughout the cross-section.

- Designed to meet FAA airfield marking requirements; brochure references FAA Advisory Circular guidelines and ICAO standards.

- Heat-applied, withstands jet blast, snow clearing, rubber removal treatments, and the daily operational demands of an active airfield.

- 5 standard colours (NOT just white and yellow): Red, White, Yellow, Black, Pink.

- Applied by certified crews — installation efficiency reduces runway / taxiway closure time vs. hand-painted multi-colour markings.

- Manufactured in an ISO 9001:2015 certified facility.

## Technical specifications

| Application | Taxiways, ramps, aprons, gates, vehicular airside roads — NON-RUNWAY |
| --- | --- |
| Material | Preformed thermoplastic with retroreflective glass beads embedded throughout cross-section |
| Colours | 5 standard: Red, White, Yellow, Black, Pink |
| Compliance reference | FAA Advisory Circular (AC 150/5340-1 series) and ICAO standards |
| Manufacturing | ISO 9001:2015 certified |
| Installation | Heat application by certified crews |
| Thickness | [Not stated in AirMark brochure reviewed — VERIFY against TDS or PPG technical sheet] |
| Cure / open-to-traffic | [Not stated in AirMark brochure reviewed — VERIFY] |
| Coverage rate | [Not stated in AirMark brochure reviewed — VERIFY] |
| Retroreflectivity (mcd/m²/lux) | [Not stated in AirMark brochure reviewed — VERIFY] |

## Colours / palette

5 standard colours: Red, White, Yellow, Black, Pink. Custom colours [VERIFY] — not stated in the brochure reviewed.

## Applications & ideal use-cases

- Taxiway centrelines, hold-short markings, designators

- Apron and ramp markings

- Helipad markings

- Gate and stand markings

- Vehicular airside roadway markings (service-road striping, intersection markings)

- Ground-vehicle parking and traffic-flow markings

## Document library

Current document set for this product. Proposed clean titles below — fixes duplicate / generic naming on the current site.

| Document title | Type | File |
| --- | --- | --- |
| AirMark Brochure (English) | Brochure | AirMark/AirMark_Brocure.pdf |
| AirMark Brochure (French) | Brochure (FR) | AirMark/AirMark-Brocure-FR.pdf |
| PreMark for Airports (landside thermoplastic) | Brochure | PreMark - Groundside/PreMark-for-Airports.pdf |
| WB Airfield Paint (waterborne, runway-capable) | TDS | Airfield Paint/WB-Airfield-Product-Data.pdf |
| Traffic Paint for Airfields (English) | Brochure | Airfield Paint/TrafficPaint-for-Airfields.pdf |
| Traffic Paint for Airfields (French) | Brochure (FR) | Airfield Paint/TrafficPaint-for-Airfields-FR.pdf |

## Source notes & [VERIFY] flags

SOURCES USED

- AirMark Brochure — English (.pdf)

- PreMark for Airports — landside thermoplastic brochure (.pdf)

- WB Airfield Product Data Sheet — waterborne paint TDS (.pdf)

- Traffic Paint for Airfields brochure (.pdf)

ITEMS REQUIRING DOUG'S CONFIRMATION

[VERIFY] Current site says "white and yellow." AirMark brochure documents FIVE standard colours: Red, White, Yellow, Black, Pink. Update the website palette.

[VERIFY] Thickness, cure time, coverage rate, retroreflectivity values: NONE of these were stated in the AirMark brochure reviewed. The TDS for AirMark specifically (separate from the airfield paint TDS) was not present in the docs folder. Source these directly from PPG before publishing on the website.

[VERIFY] Three distinct products are sometimes conflated in the AirMark folder. Recommend reorganizing the document library by product (AirMark / PreMark for Airports / WB Airfield Paint), since the website's Resources page reads from this folder.

[VERIFY] Runway vs. non-runway: Doug's note confirmed AirMark is NON-RUNWAY only. Runway markings are the territory of WB Airfield / Traffic Paint for Airfields. Make sure the website page does not imply AirMark is used on runways.

[VERIFY] FAA Advisory Circular: the brochure prints "AC 150/5349-1," which appears to be a typo. The standard current AC is AC 150/5340-1 (most recent revision is L). Confirm correct AC reference before quoting it on the site.

CURRENT SITE COPY FLAGGED AS INACCURATE / EMBELLISHED

[VERIFY] Current site lists only "white and yellow" colours. AirMark brochure lists 5 standard colours.

[VERIFY] Current site copy doesn't distinguish AirMark from PreMark for Airports or WB Airfield Paint. These are three different products in the same folder. Each should have a clear identity if any of them are to appear on the website.

Approved for website use by: ______________________________   Date: ______________