SPECIALTY — REGULATORY PAVEMENT MARKINGS

# PreMark

Manufacturer: Ennis-Flint® by PPG (PPG Traffic Solutions). PreMark® and ViziGrip® are registered trademarks; PreMarkXF™ is a trademark. HUBSS is the Canadian distributor.

PURPOSE OF THIS DOCUMENT

Source-of-truth reference for HUBSS website product copy. Replaces existing website descriptions that have been flagged as inaccurate or embellished. Doug Bain to review, correct, and sign off — every claim on the website must be supported by a fact listed in this document.

## What it is

Pre-cut preformed thermoplastic regulatory pavement markings — arrows, stop bars, yield triangles, school-zone legends, bike-lane pictograms, accessible parking symbols, crosswalk lines, rumble strips. Heat-applied with a propane torch, with no surface preheating required. Lasts 6 to 8 times longer than paint.

PreMark is offered in four manufacturing finishes (BD, VG, NB, SK) that determine surface texture and retroreflectivity. PreMarkXF is the cold-weather variant with no minimum road or ambient temperature requirement.

## Key features

- Heat-applied via industrial propane torch — drive-on immediately after installation.

- Made with up to 60% recycled material (sustainability claim from manufacturer brochure).

- Impervious to oil and gasoline.

- Manufacturing finishes: BD (Beaded — full retroreflectivity), VG (ViziGrip — beads + anti-skid for crosswalks/bike/accessibility), NB (Non-Beaded — for reversible arrows), SK (Skid-Only — for coloured preferential lanes).

- PreMarkXF cold-weather variant: "No minimum road or ambient temperature requirements for application."

- 24-month shelf life (PreMark); 2 years (PreMarkXF).

- Manufactured in an ISO 9001:2015 certified facility.

- ADA-compliant pedestrian and wheelchair-friendly surface.

## Technical specifications

| Material | Preformed thermoplastic — high-quality resin, binder, aggregate; up to 60% recycled material |
| --- | --- |
| Manufacturing finishes | BD (Beaded), VG (ViziGrip), NB (Non-Beaded), SK (Skid-Only) |
| Standard thickness | [Not explicitly stated in TS005 or PreMarkXF Brochure reviewed — site claims 125 mil / 90 mil ViziGrip — VERIFY against a PPG PreMark spec sheet] |
| Installation | Industrial propane heat torch — no surface preheating required |
| Application temperature (PreMark) | [Not stated in TS005 — VERIFY] |
| Application temperature (PreMarkXF) | No minimum road or ambient temperature requirements |
| Retroreflectivity (BD / VG variants) | Factory-applied surface beads + intermix glass beads, exposed as surface wears |
| Skid resistance | VG and SK variants carry anti-skid elements; BD relies on bead surface profile |
| Sealer | Recommended on concrete and for large multi-coloured / colorized-lane installations |
| Substrate | Asphalt and concrete (sealer recommended for concrete) |
| Shelf life | 24 months (PreMark) / 2 years (PreMarkXF) |

## Colours / palette

10 standard regulatory colours: Black, Blue, Purple, Red, Grey, Green, Orange, White, Bike Lane Green, Yellow.

## Applications & ideal use-cases

- Turn arrows, stop bars, yield triangles

- School zone legends and school-area markings

- Bike pictograms and lane symbols

- Accessible parking symbols (ISA — International Symbol of Accessibility)

- Crosswalk ladder lines and continental crosswalk patterns

- Rumble strips and traffic-calming legends

- Coloured preferential lane symbols (SK variant)

- Reversible arrows (NB variant)

## Document library

Current document set for this product. Proposed clean titles below — fixes duplicate / generic naming on the current site.

| Document title | Type | File |
| --- | --- | --- |
| Technical Data Sheet (TS005) | TDS | TS005_PreMark_220204.pdf |
| PreMarkXF Brochure (cold-weather variant) | Brochure | PreMarkXF-Brochure.pdf |

## Source notes & [VERIFY] flags

SOURCES USED

- PreMark TS005 Technical Data Sheet (.pdf)

- PreMarkXF Brochure (.pdf)

ITEMS REQUIRING DOUG'S CONFIRMATION

[VERIFY] Standard thickness — current site claims 125 mil standard and 90 mil ViziGrip. NEITHER value is explicitly stated in the TS005 sheet or PreMarkXF brochure reviewed. The 125 mil / 90 mil split is plausible (it matches the TrafficPatternsXD transverse line stripe thicknesses) but should be confirmed against a PreMark-specific spec sheet from PPG before being published.

[VERIFY] Application temperature for standard PreMark — TS005 does not state a minimum. PreMarkXF is the cold-weather variant. Confirm whether standard PreMark has a documented minimum (e.g., 45°F like other Ennis-Flint thermoplastics) or whether PreMarkXF should be the default product called out for Canadian conditions.

[VERIFY] Site lists "intersection-grade glass bead surface." Only the BD and VG manufacturing finishes carry surface beads — NB and SK do not. Recommend clarifying that retroreflectivity depends on finish selection.

[VERIFY] "Lower total cost of ownership than traditional marking methods" — defensible based on the 6–8× life claim, but no specific TCO calculation is in the docs. If the website wants to quote a number ("X% lower TCO"), source it.

CURRENT SITE COPY FLAGGED AS INACCURATE / EMBELLISHED

[VERIFY] Site spec lists "125 mil standard / 90 mil ViziGrip option" as a thickness fact. The PDFs reviewed do not state these numbers. ViziGrip (VG) is a manufacturing finish (surface beads + anti-skid), not a thickness option per se. Verify the actual thickness numbers from a current PPG PreMark spec sheet before re-using these figures.

Approved for website use by: ______________________________   Date: ______________